Mesh Connectivity Layer Installation and Configuration

# Contents

Mesh Connectivity Layer Installation and Configuration	1

Contents	1

Before You Install…	3

I. Hardware Installation	3

General Information	3

Hardware Installation on a Desktop Computer	4

Hardware Installation on a Notebook Computer	4

II. Software Installation	5

IIA. Automated Software Installation	5

IIB. Manual Software Installation	6

III. System Configuration	6

IIIA. Configuration for Automated Installation	6

Assigning SSIDs to the Adapters	6

Disabling the Firewall (optional)	11

Setting Internet and Server Addresses	12

IIIB. Configuration for Manual Installation	14

Setting TCP/IP Bindings	14

Setting SSIDs and Modes	15

Setting Internet and Server Addresses	15

IV. Confirming Installation	16

blank page

# Before You Install…

The Mesh Connectivity Layer (MCL) runs on Windows XP. This installation assumes Windows XP and all XP updates are installed.

You can choose a manual or automated installation. Automated installation performs most of the configuration steps for you.

Automated installation requires Service Pack 2 Release Candidate 2 (RC2) or later and .NET Framework 1.1. (If a previous version of .NET Framework is installed, it can be used without updating.)

To install Windows XP Service Pack 2 RC2, browse to
http://www.microsoft.com/technet/prodtechnol/winxppro/sp2preview.mspx.
Follow the installation instructions.

To install .NET Framework 1.1…

- Go to www.microsoft.com and click Downloads.

- Under Most Popular Downloads, click Next 45 and locate .NET Framework Version 1.1 Redistributable Package.

- Click the link and follow the installation instructions.

Whether you peform a manual or automated installation, you go through the same thee basic steps.

- Install the hardware.

- Install the software.

- Configure the system.

# I. Hardware Installation

## General Information

MCL can control a single wireless adapter operating at one frequency, or multiple adapters operating at multiple frequencies.

These instructions assume you’re installing two adapters. The adapters must be from different manufacturers, because no manufacturer’s drivers currently permit running two or more identical adapters in the same computer.

The hardware adapters must be installed and working before the software is installed.

If you are installing only one adapter, simply skip the instructions for installing and configuring the second one.

If the hardware adapters are already installed, you can skip to Software Installation.

## Hardware Installation on a Desktop Computer

In addition to the hardware adapter (or adapters), you’ll need the manufacturers’ driver-installation CDs.

- Shut down Windows. If the computer has a “hard” power switch (usually on the back, near the power cord), turn it off, too.

- Open the computer and install the adapters in unused PCI slots.

- Close the computer, turn it on, and log on to Windows.

During restart, Windows should locate the new cards and run the Found New Hardware wizard. The wizard will prompt you to connect to Windows Update. Select “No, not this time” and click Next.

You’ll be prompted to insert the installation CD. Insert either one and close the tray. Do not click Next. The wizard searches the CD for the necessary installation files. If it can’t find them, it’s because you used the CD for the other adapter. No problem. Click Back, then insert the correct CD.

Once the wizard finds the correct installation files, it displays a list of drivers. Highlight the one for XP, then click Next. Once the software in installed, click Finish.

Once the drivers for the first adapter have been installed, the Found New Hardware wizard runs a second time, to handle the second adapter. Repeat the driver-installation process.

If you’re prompted to restart the computer, do so.

IMPORTANT  The vendor-supplied installation disks usually include utility programs for configuring the hardware adapters. If you are performing a manual installation, you’ll need these utilities, so you should install them.

If you are performing an automated installation, do not install this software. Running it can cause configuration problems. (Exception — the Microsoft configuration software is designed for adapters using the Atheros chipset. If your adapter uses a different chipset, you’ll need the vendor’s utilities for channel control.)

## Hardware Installation on a Notebook Computer

Adapters for notebook computers are usually PC Cards. Installation generally requires no more than inserting the card in the slot, then following the instructions for driver installation.

IMPORTANT  The vendor-supplied installation disks usually include utility programs for configuring the PCI Card. If you are performing a manual installation, you’ll need these utilities, so you should install them.

If you are performing an automated installation, do not install this software. Running it can cause configuration problems. (Exception — the Microsoft configuration software is designed for adapters using the Atheros chipset. If your adapter uses a different chipset, you’ll need the vendor’s utilities for channel control.)

# II. Software Installation

IMPORTANT  Software installation requires a local XP account with administrator privileges.

## IIA. Automated Software Installation

The MCL software is supplied on a CD-ROM or USB drive, and might or might not be auto-run. To install the software…

- Insert the USB drive in a USB port, or the CD-ROM in a CD or DVD drive. If the installation package is auto-run, the Microsoft Wireless Mesh Setup wizard will run.

If the setup wizard does not auto-run, click on My Computer and navigate to the appropriate drive. You’re looking for an icon labelled “mesh.” Double-click it to start installation.

- In the Microsoft Wireless Mesh Setup wizard, click Next.

- Select a Complete setup. Click Install.

You might get multiple “This software has not passed Windows Logo testing” messages. If so, click Continue Anyway.

- Click Finish.

When installation is complete, open the Control Panel and run Network Connections. You should see a new connection, Mesh Virtual Adapter. (This is the MCL.)

You should also see one or two Wireless Network Connection icons, depending on how many hardware adapters you installed. (Their default identifiers are Wireless Network Connection and Wireless Network Connection 2.) Newly installed adapters will usually be “disabled.”

## IIB. Manual Software Installation

Manual software installation consists of running the mcl.exe utility to install the MCL virtual adapter software.

The installation software and files are located in the folder (on either the USB drive or CD ROM) d:\mcl\bins\release (where d: is the drive letter).

Open a Command Prompt window and browse to this directory. Type the following command line and press enter.

mcl install d:\mcl\bins\release

Installation should take only a few seconds.

If you now open Network Connections in the Control Panel, you should see icons for the two new adapters, plus the MCL virtal adapter.

# III. System Configuration

## IIIA. Configuration for Automated Installation

### Assigning SSIDs to the Adapters

IMPORTANT  If this computer is the first mesh node in the network, you need to create a new, unique SSID for each hardware adapter. If you are adding this computer to an existing network, you’ll assign an existing SSID to each hardware adapter.

Click Start, then select All Programs. Select Microsoft Wireless Mesh, then click Wireless Network Configuration.

On some systems, the Windows Wireless Zero Configuration service may not be enabled by default, or some wireless adapter client utilities might have turned it off. If this occurs, the following message is displayed. Follow the instructions to restart the service. (Administrative Tools is accessible from the Control Panel.) Then start Wireless Network Configuration again.

[Error: WZC Service Not Available]
This software requires the Wireless Zero Configuration service, but the service is currently stopped or disabled. To start it, launch the Services applet in Administrative Tools, select the Wireless Zero Configuration service, set the startup type to Automatic, and click Start under Service Status.

If this is the first mesh node in a new network, you need to supply an SSID for each adapter.

- Select Wireless Network Connection from the list box at the top.

This enables settings for the adapter card labelled Wireless Network Connection under Network Connections in the Control Panel.

- Click Add.

- Enter the SSID identifier you want in the SSID box. SSID identifiers are case-sensitive — MyMeshNetwork, MYMESHNETWORK, and mymeshnetwork are three distinct SSIDs.

IMPORTANT  If the SSID identifier includes the string “mesh”, new mesh nodes having the same identifier will be automatically configured as mesh nodes. If the SSID doesn’t contain “mesh,” each participant must use this dialog box to specify that they’re part of a mesh network. If not, their connection will be treated as ad-hoc. We therefore strongly recommend that SSID identifiers include “mesh.”

- Select Automatic connection, and Mesh Network mode.

- In the Options, select the band and channel.

Don’t leave these settings at Auto. In auto mode, each adapter searches the channels for other adapters, and might choose channels other than those you want. As this is the first mesh node in the system, there are no other adapters to search for, and you must choose a specific channel.

- Click OK. The new SSID name should appear in the Preferred Networks list.

NOTE  The preceding steps performed by the Microsoft Wireless Mesh Setup wizard are the same as would be performed during a manual installation.

- The protocol and adapter bindings are set.

- SSIDs are created and assigned.

- The bands and frequencies are selected.

To add a second new SSID for a multiple-adapter installation, select Wireless Network Connection 2 from the list box at the top. (This enables settings for the adapter card labelled Wireless Network Connection 2 under Network Connections in the Control Panel.) Repeat Steps 2 through 6. Then click OK.

The second SSID must be different from the first. This is usually done by adding a digit to the first identifier — MyMeshNetwork2.

#### Assigning SSIDs for Additional Mesh Nodes

When adding new mesh nodes to an existing mesh network, the configuration procedure is the same, except you assign an existing SSID (the one created in the preceding steps) to the adapter card.

- Select Wireless Network Connection from the drop-down list box.

- Highlight the SSID you want in the Available Networks box.

IMPORTANT  If the Available Networks box is empty, or doesn’t list networks that ought to be present, click Refresh.

- Click Make Preferred.

- The SSID is added to the Preferred Networks box. If more than one network appears in this box, be sure they’re listed in the order you want them searched for.

If you have a second adapter card, repeat this process for it by selecting Wireless Network Connection 2 from the drop-down list box at the top. Repeat steps 2 through 4, selecting a different SSID. Then click OK.

### Disabling the Firewall (optional)

When the Windows XP SP2 is installed, firewalls for all network adapters are enabled by default.

- Open the Control Panel.

- Right-click the Mesh Virtual Adapter icon, then select Properties.

- Select the Advanced tab.

- Click the Settings button in the Windows Firewall section.

- Click the No Firewall radio button.

- Click OK twice.

### Setting Internet and Server Addresses

- Open the Control Panel.

- Right-click the Mesh Virtual Adapter icon, then select Properties.

- Scroll down the Items list to Internet Protocol (TCP/IP), then highlight it.

- Click Properties.

- Select “Use the following IP address,” then enter the IP address and subnet mask.

The subnet mask is usually 255.255.255.0. The IP address should be one that is not routable on the Internet. The most-common of these take the form 10.10.nnn.nnn and 192.168.nnn.nnn.

If your network supports DHCP, you can select “Obtain an IP address automatically” and let the DHCP set the address for you.

If your system has a DNS server and you want access to it, mark the “Use the following DNS Server Addresses” checkbox and check with the administrator for the correct values.

- Click OK twice.

The MCL virtual adapter is now installed and configured.

## IIIB. Configuration for Manual Installation

When installation is performed manually, additional configuration steps are required.

### Setting TCP/IP Bindings

The principal difference between an ad-hoc network and a mesh network is in the bindings. The hardware adapters are bound to the MCL virtual adapter, and to nothing else.

First, unbind any “wired” local-area connection from MCL.

- Open the Control Panel. Run Network Connections.

- Right-click the “Wired” Local Area Connection. Select Properties.

- Uncheck the MCL (Mesh Connectivity Layer) Protocol checkbox.

- Click OK.

Second, bind the wireless adapters to MCL only.

- Open the Control Panel. Run Network Connections.

- Right-click the Wireless Network Connection icon. Select Properties.

- Uncheck all the bindings except MCL (Mesh Connectivity Layer) Protocol.

- Click OK.

- If you have more than one hardware adapter, repeat these steps for Wireless Network Connection 2.

Third, unbind the mesh virtual adapter from MCL.

- Open the Control Panel. Run Network Connections.

- Right-click Mesh Virtual Adapter. Select Properties.

- Uncheck the MCL (Mesh Connectivity Layer) Protocol checkbox.

- Click OK.

### Setting SSIDs and Modes

The wireless hardware adapters are usually supplied with configuration utilities. Install them if they are not already installed. Then set the following items.

- Create new SSIDs for the hardware adapters (if this is the first node in the mesh), or assign the correct existing SSIDs to the adapters (if this node is the second node or an additional node in the mesh). Identifiers including the string “mesh” are strongly preferred.

- Pick the band and channel (frequency) for each hardware adapter.

- Set the network mode to ad-hoc.

- Disable WEP/Security.

### Setting Internet and Server Addresses

- Open the Control Panel. Run Network Connections.

- Highlight Internet Protocol (TCP/IP) in the list of protocols.

- Click Select, then Properties.

- Mark the “Use the following address” checkbox. Enter the appropriate values.

The subnet mask is usually 255.255.255.0. The IP address should be one that is not routable on the Internet. The most-common of these take the form 10.10.nnn.nnn and 192.168.nnn.nnn.

If your system has a DNS server and you want access to it, mark the “Use the following DNS Server Addresses” checkbox and check with the administrator for the correct values.

- Click OK twice. Then quit Network Connections.

The MCL virtual adapter is now installed and configured.

# IV. Confirming Installation

Once two or more mesh nodes are operating, they should form a network within several seconds.

- Try pinging the first mesh node that was installed. Open a Command Prompt. Type “ping,” followed by the node’s IP address. A response indicates that the mesh node can communicate in both directions.

- If your network has an Internet gateway, run Internet Explorer and see if you can browse the Web.

The TTCP utility can measure the throughput between two nodes. For automated installtions, it’s in c:\Program Files\Microsoft Wireless Mesh. With a manual installation, it’s in the directory with MCL binaries. For this release, that’s c:\mcl\bins\release.

- On the receiving node, open a Command Prompt and browse to the directory with TTCP.

- Type ttcp –r and press enter.

The receiving node is now ready to receive packets from the sending node.

NOTE  Typing ttcp with no arguments displays TTCP usage.

- On the sending node, open a Command Prompt and browse to the directory with TTCP.

- Type the following

ttcp – t –L<secs> <address>

and press enter. <secs> is the number of seconds you want the test to run (such as 60), and <address> is the address of the receiving node (such as 10.10.11.8).

At the end of the test, both the transmitting node and receiving nodes will display route usage and the average throughput.

sending node

ttcp-t: local 10.10.11.201 -> remote 10.10.11.12
ttcp-t: buflen=8192, duration=120s, align=16384/+0, port=5001 tcp
ttcp-t: done sending, nbuf = 33511
ttcp-t: 274522112 bytes in 119986 real milliseconds = 17874 Kbps
ttcp-t: 33511 I/O calls, msec/call = 3, calls/sec = 279, bytes/call = 8192
    Route Usage:
        234577: f4-cb-8e-f7-ba-1e/0/4 0.00-33Mbps-56 28-76-4e-4b-d9-17/6/0
      num routes 1 total packets 234577 num changes 2
      average path length 1.00 hops

receiving node

ttcp-r: local 10.10.11.12 <- remote 10.10.11.201
ttcp-r: buflen=8192, align=16384/+0, port=5001 tcp
ttcp-r: 274522112 bytes in 119991 real milliseconds = 17873 Kbps
ttcp-r: 33836 I/O calls, msec/call = 3, calls/sec = 281, bytes/call = 8113
    Route Usage:
        117367: 28-76-4e-4b-d9-17/0/6 0.00-27Mbps-56 f4-cb-8e-f7-ba-1e/4/0
      num routes 1 total packets 117367 num changes 0
      average path length 1.00 hops

Barring hardware failure (which is easily diagnosed by swapping-out adapters), the most-likely problem is incorrect configuration. Check the following settings.

- SSIDs

- bands

- channels

- bindings

- IP address for MCL virtual adapter

- subnet mask for MCL virtual adapter